Air Quality Data Analysis for Nebraska

Client: UNMC Water, Climate and Health Working Group

Date: February 3, 2026

- Introduction and Background

Air quality is an important factor influencing public health, particularly for sensitive populations such as children, older adults, and individuals with respiratory conditions. The UNMC Water, Climate and Health Working Group requested an analysis of air quality data collected across Nebraska to better understand current air pollution levels and identify potential areas of concern.

This project analyzes air quality data collected from AirPurple sensors located throughout Nebraska. The analysis focuses on three primary pollutants: volatile organic compounds (VOC), particulate matter less than 2.5 micrometers in diameter (PM2.5), and particulate matter less than 10 micrometers in diameter (PM10.0). In addition, the project examines how environmental factors including temperature, humidity, sensor altitude, and geographic location may influence air quality patterns.

The objective of this work is to summarize air quality conditions, identify potential pollution hotspots, determine when and where extreme pollution events occurred, and assess whether air quality meets EPA National Ambient Air Quality Standards (NAAQS). The results provide UNMC with a clearer understanding of air quality trends that may be relevant to public health in Nebraska.

- Methods

  - Data Source

The analysis was conducted using a provided AirPurple dataset containing daily air quality measurements from multiple sensor locations across Nebraska. The raw data were provided as a CSV file and imported into Python for analysis.

  - Data Processing

All data analysis was performed using Python and the pandas library. The dataset was first inspected to verify column names, data types, and overall structure. Key variables such as temperature, humidity, altitude, VOC, PM2.5, and PM10.0 were confirmed to be in numeric format. Missing or invalid values were identified and excluded from calculations where necessary.

  - Data Organization

Data were grouped by sensor name to allow comparison of air quality across different locations. Counts of observations per sensor were calculated to confirm data coverage and ensure that each sensor had sufficient data for analysis.

  - Statistical Analysis

For each sensor location, summary statistics including mean, median, minimum, and maximum values were calculated for VOC, PM2.5, and PM10.0 concentrations. These statistics were used to identify the five locations with the highest mean and median pollutant levels. Daily maximum pollutant values were also calculated to determine when and where the highest pollution events occurred.

  - Environmental Categorization

Humidity values were categorized into low (<50%), high (50–80%), and very high (>80%) ranges. Temperature values were categorized as below freezing (<32°F), cool (32–50°F), warm (51–70°F), and hot (>70°F). Pollutant concentrations were compared across these categories to evaluate potential environmental influences on air quality.

  - Altitude Analysis

Air quality measurements were grouped by sensor altitude, and average pollutant concentrations were calculated to assess whether elevation influenced observed air quality values.

  - AQI Evaluation

PM2.5 and PM10.0 values were compared to EPA Air Quality Index (AQI) thresholds for Unhealthy Sensitive Groups. Dates and locations exceeding these thresholds were identified and summarized.

- Results and Discussion

The results show that air quality levels vary by location across Nebraska. When comparing average and median values, Broken Bow and #16 – Richardson County Courthouse consistently appeared among the top locations with the highest PM2.5 and PM10 concentrations. These locations had much higher mean values compared to their median values, which suggests that extreme pollution events strongly influenced the averages. This means air quality at these sites is usually lower but occasionally experiences very high spikes.

The highest PM2.5 and PM10 values both occurred at #16 – Richardson County Courthouse on February 18, 2025. This indicates a short-term pollution event rather than a long-term trend. Similar extreme values were also observed at Broken Bow on multiple dates. Based on exploratory research, these high values could be related to wildfire smoke transported into Nebraska, dust from agricultural activities, or weather conditions that limited air movement and trapped pollutants near the ground.

The AQI analysis showed many days where PM2.5 and PM10 levels exceeded EPA thresholds for unhealthy air quality for sensitive groups. Broken Bow had the highest number of risk days, followed by Richardson County Courthouse and the Southeast District Health Department location. This suggests that some communities may experience repeated periods of poor air quality, which could be a concern for people with asthma, heart conditions, or other respiratory issues.

When comparing air quality with environmental conditions, higher pollution levels were more common during warmer temperatures and higher humidity conditions. Warm air can increase chemical reactions in the atmosphere, while high humidity can allow particles to remain suspended in the air longer. These factors may help explain why pollution events were more severe during certain times.

The altitude analysis did not show a clear pattern between elevation and air quality. High pollutant concentrations occurred at both low and high altitudes. For example, some lower-altitude sensors showed extremely high PM values, while some higher-altitude sensors reported relatively low averages. This suggests that local sources and short-term events have a much stronger influence on air quality than sensor altitude alone.

- Conclusions

This project analyzed air quality data from AirPurple sensors across Nebraska to identify pollution patterns and possible public health concerns. The results showed that air quality varies by location, with Broken Bow and the Richardson County Courthouse experiencing the highest PM2.5 and PM10 values and the greatest number of AQI risk days. Most high pollution levels occurred during short-term events rather than being constant over time. Temperature and humidity appeared to influence air quality, while sensor altitude did not show a clear effect. Overall, air quality in Nebraska is generally acceptable but can experience occasional extreme pollution events that may affect sensitive populations.

- References

US EPA PM2.5 Air Quality Standards Interactive map by PurpleAir. PurpleAir. (n.d.). https://map.purpleair.com/air-quality-standards-us-epa-aqi

Environmental Protection Agency. (n.d.). Particulate Matter (PM) Basics. EPA. https://www.epa.gov/pm-pollution/particulate-matter-pm-basics

Smoke and Air Quality. Smoke and Air Quality | DWEE NE. (n.d.). https://dee.nebraska.gov/smoke-and-air-quality